ELECTRUM

# Bubbler

Product definition

2026-10-28

Documents in this file: Executive Summary, Open Questions, Explore Notes, Team, High Level Design, Component Arrangement, System Description, Gate Checklist.

# Contents

Open in Word and choose Yes when asked to update fields; this line becomes the table of contents.

# Executive Summary

| Field | Value |
| --- | --- |
| Date | 2026-10-28 |
| Author | Product lead (Electrum workflow, hands-free run) |
| Status | Draft |

## 1. Business Rationale

The need: Large soap bubbles, half a metre across, hold a crowd in a way small ones never do, and families, street performers and event organisers want them without the skill it takes to make them by hand. Doing it by hand means getting film thickness, airflow and timing right every single time; most people cannot, and the ones who can get tired.

Competition and alternatives: Rotating-disk consumer bubble machines ($15–40) run a fixed fan over a spinning wheel of small loops and make 20–50mm bubbles with a high pop rate and no adaptation to wind or solution; they fill the air but never impress. Hand-held giant-bubble wands and tri-string kits make big bubbles but need a practised operator and stop when the operator does. Professional stage bubble equipment exists at prices and sizes no family buys. Nothing in the consumer range makes large bubbles unattended.

Value proposition: For under $50, Bubbler makes large, stable bubbles on its own, at a steady pace, and keeps making them as the wind picks up and the solution ages. The whole difference is one $0.80 sensor and the firmware behind it, on a bill of materials of about $12.80 at 10k units.

## 2. The Solution

Bubbler is a battery-powered outdoor machine the size of a cake tin. You fill its open tray with bubble solution, put in four AA batteries and press one button. A 160mm loop dips into the tray, lifts upright, and a slow, gentle fan inflates the film into a bubble up to half a metre across that releases on its own; then it dips again. A force sensor on the loop's arm feels each bubble growing and the machine adjusts its own fan speed and timing from one bubble to the next, so it gets better over the first few cycles and copes with a light breeze. It runs for at least two hours on a set of batteries. V1 is standalone: no app, no Wi-Fi, no account; two buttons and four lights are the whole interface. It is a toy-grade consumer product, not stage equipment.

## 3. Domains: Challenges and Deliverables

| Domain lead | Hardest challenges | Required deliverables at the gate |
| --- | --- | --- |
| Product | Prove the adaptive loop is worth its cost against fixed settings (risk: only a 20% gain); decide toy vs. general-product classification (open question #6); keep the holiday retail window with a 30-week critical path | Approved scenarios; decision log with explicit consequences (gate gap #70); classification decision |
| Electrical / electronics | Resolve 50–200 mN on a cheap foil gauge through an HX711 with the motor running and the product outdoors; meet FCC 15B / CE EMC with a 25 kHz PWM and a 48 MHz MCU on a 2-layer board | Released schematic and layout; gauge signal-chain bench results against the 5 mN target; measured EVT power budget; EMC pre-scan plan |
| Firmware | Classify every cycle from the force curve (success, pop, no-film, partial) and converge the five-parameter hill-climb in 5–10 cycles while telling "wind changed" from "solution degraded" | Nine module skeletons; classifier validated on recorded curves; convergence shown on the bench; FW version struct in flash (gate gap #34) |
| Mechanical / industrial design | Keep the film intact through the 175° rotation (no jerk, no flex); produce 0.5–2 m/s laminar flow across a 160mm opening from a 40mm blower through an L-bent duct; seal the shaft where it crosses from the wet tray to the dry electronics | Tooling-ready CAD for base, U-rim and vat; wand, shaft, bearing and seal drawings; duct airflow measurement; gauge location and loop material decided |
| Manufacturing / NPI | Hold BOM under $15 with a 5k-unit motor MOQ and three moulds; make strain-gauge bonding and factory tare a repeatable line step; fit EVT, DVT, PVT and certification into 30 weeks | Tooling quotes; gauge bonding and calibration work instruction; DVT and PVT build plans; production test spec |

# Open Questions

Created by the hands-free resume run of 2026-10-28, which added the team, the §4 handoff split and the executive summary to an existing complete run. Rows the run assumed because nobody was present to decide.

| # | Question | Source | Phase | Status |
| --- | --- | --- | --- | --- |
| 1 | Team roster: five leads as proposed (Product, Electrical, Firmware, Mechanical / ID, Manufacturing / NPI), UX merged into Product, compliance split between Electrical and Manufacturing | Phase 1 team step | Phase 1 | Assumed: recommended roster kept (hands-free) |
| 2 | Competition and alternatives: rotating-disk consumer bubble machines and hand-held giant-bubble wands, as §1 of the System Description states; no named brands | Phase 1 envelope | Phase 1 | Assumed: taken from the existing System Description |
| 3 | Who signs off the executive summary's business figures (sub-$50 retail, 3.5× markup, 10k units) | Executive summary | Floating | Open |

# Explore Notes

## Product Summary

An automated large-bubble machine that produces soap bubbles up to ~500mm diameter and actively optimizes bubble size and stability using force-sensing feedback. A single large loop (~160mm opening) dips horizontally into an open vat of soap solution, rotates to vertical, and a gentle fan inflates the film into a large bubble. A strain gauge on the wand arm tracks the inflation curve in real time — firmware adjusts fan speed, dip duration, and blow ramp rate to maximize successful large bubbles. Consumer product, outdoor/portable, battery-powered, targeting sub-$50 retail at 10k+ unit volumes.

## Product Classification

Electromechanical. The product moves air (fan/blower at low speed for gentle inflation), rotates a mechanical wand arm (dip-and-present mechanism), and uses closed-loop sensor feedback to optimize the process. Two motors, an open liquid vat, and a force-sensing feedback loop.

## HW/SW Boundary Analysis

| Domain | Hardware | Firmware | Software (App) |
| --- | --- | --- | --- |
| Bubble production | Fan motor (low-speed blower), wand arm with loop, pivot motor/actuator | Fan PWM ramp control, wand dip/rotate timing | N/A |
| Auto-optimization | Strain gauge or load cell on wand arm pivot, HX711 ADC | Inflation curve analysis: slope = growth rate, peak = size, sudden drop = pop vs. clean detach. Adaptive control adjusts fan ramp, dip time, pause-before-blow. | Optional — display optimization stats, let user set target bubble size |
| Solution management | Open vat (user fills manually), no pump or wick | Optionally sense solution level (capacitive) | Low-solution alert |
| User interface | Power button, mode dial or buttons, LEDs | Mode state machine, LED feedback | Optional BLE companion app |
| Power | LiPo or 4×AA, motor drivers | Battery monitoring, motor power management | Battery status |

Primary sensing approach: Force on the wand arm.

At 500mm bubble scale, the forces on the wand arm during inflation are significant and measurable:

Film weight + drainage: A large soap film (~0.8 m² surface area, 1–4 µm thick) weighs 0.8–3.2g. Solution drains to the bottom, concentrating weight.

Aerodynamic drag: As the bubble inflates to 500mm, its cross-section catches the gentle airflow. Even at low fan speed, this adds measurable force.

Total force on wand arm: Estimated 50–200 mN (5–20 grams-force), building over several seconds.

A cheap strain gauge ($0.30) + instrumentation amplifier or HX711 ($0.50 at volume) on the wand arm pivot can track the full inflation curve.

Why force sensing beats other options at this scale:

IR break-beam would need components spanning the 160mm loop opening, and can't track the bubble as it inflates 250mm+ beyond the loop. Bulky and gives only binary "present/absent."

Pressure sensing in the duct is overwhelmed by outdoor wind.

Motor current sensing signal is too small relative to baseline motor draw.

Force sensing is fully internal (no external protrusions), compact (embedded in pivot), gives a continuous inflation curve over seconds, and distinguishes pop (abrupt drop mid-rise) from clean detach (smooth peak then gradual drop).

The firmware optimization objective function:

Maximize: successful detach rate (bubbles that inflate to target size and release cleanly)

Minimize: premature pops (film burst during inflation)

The force curve signature tells the firmware everything: inflation rate (fan too fast?), final size (fan too slow?), failure mode (pop = too aggressive, slow drain = dip too brief).

## Physical Architecture

### Physical Function

Inflates large soap bubbles (~500mm diameter) by gently blowing air through a soap-film-coated loop. The loop dips horizontally into an open vat of soap solution, soaks to build a thick film, then rotates 90° to vertical and presents the film to a slow, directed airstream.

### Mechanical Subsystem

Blower fan: Low-speed fan or ducted blower (60–80mm). Must produce gentle, laminar-ish airflow — NOT high-velocity turbulent air. Large bubbles need very low airflow velocity (~0.5–2 m/s at the loop). A large slow fan is better than a small fast one. Variable speed via PWM.

Wand arm with loop: A rigid arm (~200mm) with a ~160mm diameter loop at the end. The arm pivots at its base — horizontal position for dipping, vertical position for blowing. A geared DC motor or servo drives the pivot. The rotation must be smooth and controlled — a jerk or vibration during rotation will break the film.

Open vat: A wide, shallow trough the user fills with soap solution. The loop submerges fully when horizontal. No pumps, no wicks, no sealed reservoir. The vat must be wide enough for the loop (~180mm minimum internal width) and deep enough for full submersion (~20–30mm depth). Removable or integrated with a pour spout for filling.

Cam or linkage (optional): If a single motor drives both the dip/rotate and presents to the fan, a cam profile could sequence: dip → soak → rotate → blow → rotate back → dip. But a separate servo for the arm is simpler and allows firmware to control timing independently.

### Structure and Load Paths

Injection-molded plastic shell, unified structure/enclosure.

The product has a roughly upright form factor: vat at the bottom, fan at the back, wand arm pivoting from the middle, loop extends above/in front.

Fan mounts in a molded duct in the rear, aimed forward at the vertical loop position.

Wand arm pivot mounts mid-body. Motor or servo below the pivot, inside the dry zone.

Vat sits in the base — open top, removable or integrated.

Battery compartment in the base or rear, sealed from the wet zone.

The wand arm is the main dynamic load — small forces (< 1N), low speed. No structural concerns beyond keeping the pivot smooth and the arm stiff enough not to flex excessively (which would corrupt the strain gauge signal).

### Working Media and Interfaces

Air: Ambient, drawn from rear, directed gently forward through a duct/nozzle. Low velocity. No pressurization.

Soap solution: Aqueous with surfactant + glycerin/polymer additives for large-bubble stability. Open vat — user fills it. Solution contacts the vat, wand arm lower section, and loop. Materials: PP or ABS (soap-resistant). The open vat means transport leakage is the user's problem — provide a snap-on lid for the vat.

Outdoor exposure: Splashes from bubbles, rain, sun. Electronics must be isolated from the wet zone (vat and wand area). IPX4 for the electronics enclosure. The vat and wand arm can be fully wet — they're just plastic.

### Physical-Electronic Interface

Fan motor driver: MCU PWM → MOSFET → fan motor. Gentle ramp control (not on/off — gradual spin-up to avoid bursting the film with an air blast).

Wand arm motor/servo: MCU PWM → servo or MCU PWM → H-bridge → geared DC motor. Position control for dip (horizontal) → present (vertical) → return.

Strain gauge on wand arm: Strain gauge bonded to wand arm near pivot → instrumentation amp or HX711 ADC → MCU. Sampled at ~10–50 Hz (bubble inflation takes seconds). Gives continuous force curve.

Solution level sensor (optional): Capacitive sense on vat wall. Simple threshold: solution present / low.

Battery voltage: ADC channel for fuel gauge.

## Relevant Skill Areas

Sensors & Actuators (#10) — Core differentiator. Strain gauge integration, signal conditioning for a small force signal in a vibrating, outdoor environment.

Embedded Software & Firmware (#5) — The optimization loop. Analyzing the force curve in real time, classifying outcomes (success/pop/no-film), adjusting parameters adaptively.

Mechanical & Industrial Design (#4) — The dip-rotate-blow mechanism. Smooth wand arm motion, open vat design, wet/dry zone separation, airflow duct for gentle laminar flow.

Power Management (#9) — Two motors + MCU + strain gauge amp. Battery-powered outdoor product.

Cost & BOM Awareness (#15) — Sub-$50 retail. The strain gauge + amp adds ~$1–2 to BOM vs. a dumb bubble machine. Must justify with clearly better performance.

Manufacturing (#14) — Strain gauge bonding to the wand arm is a manual or semi-automated step. Calibration at factory (zero the strain gauge with no load).

## Key Unknowns and Questions

Strain gauge signal quality. Can a cheap strain gauge on the wand arm reliably distinguish pop vs. clean detach vs. no-film in an outdoor environment with motor vibration? Needs prototype validation. Filtering at 10–50 Hz sampling should reject motor vibration (which is higher frequency).

Optimal airflow profile for large bubbles. How should the fan ramp up? Linear? Exponential? Start very slow and accelerate as the bubble grows? This is what the optimization loop must learn.

Wand arm rotation speed and profile. Too fast = film breaks during rotation. Too slow = film drains before reaching the fan. What's the sweet spot? Does it depend on solution chemistry?

Solution chemistry sensitivity. How much does the optimal fan/timing profile change when the user mixes their own solution vs. using a supplied concentrate? Can the firmware adapt automatically, or does it need a "solution type" setting?

Battery life target. Fan motor (~0.5–1W) + servo (~0.5W intermittent) + MCU + strain gauge amp (~0.1W). Total ~1–2W average. 4×AA (9 Wh) → 4.5–9 hours. LiPo 3S 2200mAh (24 Wh) → 12–24 hours. What's the target?

App or standalone? The optimization works standalone. A BLE app could show the inflation curve in real time (satisfying to watch), let the user set target bubble size, and log performance over time. Worth the ~$1–2 BLE cost?

Vat size and portability. A 180mm+ wide vat is large. Can it detach from the machine body for easy filling and transport? Should the machine fold or disassemble?

## Initial Risk Areas

| Risk | Severity | Likelihood | Notes |
| --- | --- | --- | --- |
| Strain gauge signal too noisy outdoors | High | Medium | Motor vibration + wind. Mitigation: low-pass filter (inflation is slow, ~0.5 Hz signal), mechanical vibration isolation at pivot. Needs prototype. |
| Large soap films pop before inflation starts | High | Medium | Film drainage is the enemy. Mitigation: minimize time between dip and blow, optimize dip duration for thick film, gentle fan ramp. |
| Wand arm rotation breaks film | High | Medium | Jerky motion = broken film. Mitigation: servo with controlled velocity profile, or geared DC motor with soft-start. |
| Product too large/awkward for portable use | Medium | Medium | 160mm loop + vat + fan = product footprint maybe 200×200×250mm. Acceptable for "outdoor portable" but not pocket-sized. |
| Optimization doesn't noticeably outperform fixed settings | Medium | Medium | If a fixed fan speed and timing works 80% of the time, the adaptive loop only adds 20% improvement. Is that worth the sensor cost? |
| BOM exceeds $15 target | Medium | Low | MCU ($1) + strain gauge + amp ($1.50) + fan motor ($1.50) + servo ($2) + battery holder ($0.50) + shell ($2) + PCBA ($1.50) + misc ($1.50) ≈ $12. Tight but feasible. |

## Suggested Focus for High-Level Design

Wand arm mechanism: Single servo or geared motor for dip-rotate? Cam-driven or independently controlled? This is the core mechanical design.

Airflow design: Duct geometry for gentle, directed airflow at the loop position. Fan selection (size, type, speed range).

Force sensing integration: Where exactly on the wand arm? How to mount the strain gauge? What signal conditioning? Factory calibration procedure.

Control loop architecture: What parameters does firmware control (fan speed ramp, dip duration, soak time, rotate speed, blow duration)? What does the force curve tell it? What's the adaptation algorithm (PID? lookup table? gradient descent on success rate)?

Wet/dry boundary: Where does the wet zone end and the dry zone (electronics) begin? The wand arm crosses this boundary — its pivot shaft penetrates from wet to dry.

# Team

Proposed in Phase 1 from the skills map, the electromechanical classification and the envelope (consumer, outdoor/portable, budget, battery, mass production). Hands-free run: the recommended roster was kept as proposed; no roles added.

| Role | Required | Why this product needs it | Owns | Deliverables at gate |
| --- | --- | --- | --- | --- |
| Product lead | Required | Owns the sub-$50 / 10k-unit envelope, the three user scenarios (party, performer, first use) and the two error scenarios, the "large bubbles reliably" promise, and every decision this run records. Also carries UX: two buttons, four LEDs and the fill-and-go flow are not enough to justify a separate UX lead. | §1, §2, §8; high_level_design.md; open_questions.md | Approved scenarios; decision log with consequences (gate gap #70); toy-vs-general-product classification (open question #6) |
| Electrical / electronics lead | Required | One 2-layer PCBA (STM32C011, HX711, DRV8837, fan MOSFET, LDO), the strain-gauge signal chain that the whole differentiator rests on, battery monitoring, EMC for FCC 15B / CE. Also carries compliance for EMC and RoHS. | §4.1, §5, §6, §9 Regulatory (EMC) | Schematic and layout released; strain-gauge signal-chain bench results (noise floor vs. 5 mN target); power budget measured on EVT; FCC/CE pre-scan plan |
| Firmware lead | Required | Bare-metal superloop with the force-curve analyser, hill-climbing optimiser, cycle state machine, soft-start motor and fan ramps, battery monitor. The adaptation loop is the product. | §4.2 | Module skeletons for the nine modules; force-curve classifier validated on prototype data (success / pop / no-film / partial); FW version struct in flash (gate gap #34) |
| Mechanical / industrial design lead | Required | Electromechanical product: dip-rotate-blow wand mechanism, L-bent duct for 0.5–2 m/s laminar flow, three-part injection-moulded enclosure (base, U-rim, removable vat), wet/dry boundary at the shaft, IPX4 electronics compartment. Enclosure tooling is 8 of the 30 weeks on the critical path. | §4.5, §9 Mechanical and Environmental, §9 Manufactured Part Lead Times | Enclosure CAD ready for tooling; wand-arm and shaft drawings with bearing and seal spec; duct airflow test; strain-gauge mounting location decided (open question #1); loop material chosen (open question #4) |
| Manufacturing / NPI lead | Recommended | Mass production at 10k+ units: three moulds, strain-gauge bonding and factory calibration (tare) as a manual step, geared-motor MOQ 5k at 6–8 weeks, EVT/DVT/PVT builds, toy-safety (ASTM F963 / EN 71) if classified as a toy. | §9 BOM Cost, Assembly and Inspection Times, Schedule, DfX | Tooling quotes for base, U-rim, vat; strain-gauge bonding and calibration work instruction; DVT and PVT build plans; production test spec (tare, motor sweep, fan ramp) |

Not on the team, and why: App lead and Cloud / backend lead (V1 is standalone: no app, no cloud, no radio); ML / data lead (the optimiser is integer hill-climbing on five parameters, not a trained model); UX lead (merged into Product lead, see above); Security and compliance lead (no data, no attack surface; EMC/RoHS carried by Electrical, toy safety by Manufacturing / NPI).

# High Level Design

### Bubbler — High-Level System Design

Date: 2026-02-25 | Author: Electrum workflow | Status: Draft

#### What It Is

A battery-powered outdoor machine that produces large soap bubbles (up to ~500mm diameter) by dipping a 160mm loop into an open vat, rotating it to vertical, and gently inflating the film with a slow fan. A strain gauge on the wand arm pivot tracks the inflation curve in real time — firmware adjusts fan speed, dip duration, and blow ramp to maximize successful large bubbles and resist light wind (up to 4 kph). Standalone operation, physical controls only, targeting sub-$50 retail at 10k+ units.

#### Block Diagram

#### Subsystems

| Subsystem | Purpose | Domain |
| --- | --- | --- |
| Blower fan (60–80mm ducted) | Produce gentle, directed airflow (~0.5–2 m/s) to inflate the soap film into a large bubble | HW — Mechanical |
| Wand arm + 160mm loop | Dip horizontally into vat, rotate 90° to vertical, present soap film to airflow | HW — Mechanical |
| Pivot motor (geared DC or servo) | Drive wand arm rotation with smooth, controlled velocity profile | HW — Mechanical |
| Open vat | Hold soap solution; loop submerges fully when horizontal (~180mm wide, ~25mm deep) | HW — Mechanical |
| Strain gauge + HX711 ADC | Measure force on wand arm during inflation (50–200 mN range, 10–50 Hz sample rate) | HW — Sensing |
| MCU + firmware | Run optimization control loop, classify bubble outcomes, manage dip-rotate-blow state machine, drive motors | FW |
| Fan motor driver (MOSFET) | PWM-controlled gentle ramp of fan speed — no sudden air blasts | HW — Electronic |
| Pivot driver (H-bridge or servo PWM) | Position control for wand arm: horizontal ↔ vertical with soft-start/stop | HW — Electronic |
| Battery + regulator | 4×AA (6V nominal) or LiPo pack; 3.3V regulated rail for MCU and ADC | HW — Power |
| User controls (button, mode dial, LEDs) | Power on/off, mode selection (e.g., continuous / single / demo), status feedback | HW — UI |

#### Key Interfaces

| From → To | What Crosses | Protocol / Medium |
| --- | --- | --- |
| Strain gauge → HX711 | Analog differential voltage (µV–mV range) | Wheatstone bridge excitation, differential input |
| HX711 → MCU | Digitized force readings (24-bit) | Serial clock + data (DOUT/SCK), polled at 10–80 Hz |
| MCU → Fan MOSFET | Fan speed command | PWM (25 kHz), soft ramp in firmware |
| MCU → Pivot driver | Arm position / velocity command | PWM (servo) or PWM + DIR (H-bridge with encoder or timed open-loop) |
| MCU → LEDs | Status: idle / running / optimizing / low battery / error | GPIO (3–5 lines) |
| Mode dial / buttons → MCU | User mode selection, power | GPIO with debounce in firmware |
| Battery → regulators → all | Power rails: V_bat (4.5–6V AA or 7.4–11.1V LiPo), 3.3V logic | Linear or buck regulator |

#### Constraints

| Constraint | Value | Why It Matters |
| --- | --- | --- |
| BOM cost | < $15 at 10k units ($50 retail) | Rules out precision load cells, BLE modules, custom motors. Every dollar counts. |
| Battery life | ≥ 2 hours continuous operation | Average draw ~1.5W (fan ~0.8W, servo ~0.5W intermittent, MCU+sensing ~0.1W). 4×AA (≈9 Wh) gives ~4–6 hrs. LiPo 2S 2200mAh (≈16 Wh) gives ~8–10 hrs. |
| Wind resistance | Operate in side wind up to 4 kph | Fan must compensate — firmware detects wind via asymmetric force signatures and increases airflow. Duct geometry should partially shield the loop. |
| Outdoor / splash exposure | IPX4 for electronics enclosure | Open vat + bubble splashback. Electronics must be sealed from wet zone. Motors and wand arm can be wet. |
| Noise | Not a primary concern (outdoor use) | Fan and servo noise acceptable outdoors. No acoustic constraints. |
| Size | Footprint ≤ 250×250mm, height ≤ 300mm | Must be portable — carried to a park. Includes vat width (≥180mm for loop). |

#### Fundamental Hardware Problems

| Problem | Why It's Fundamental |
| --- | --- |
| Keeping the soap film intact during wand rotation from horizontal to vertical | If the film breaks during rotation, no bubble is produced regardless of how good the sensing and fan control are. Film drainage during rotation thins the film; any jerk or vibration tears it. This constrains motor choice, velocity profile, and mechanical stiffness of the arm. |
| Producing gentle, directed airflow that inflates a 500mm bubble without popping it | Large soap films need very low velocity (~0.5–2 m/s), evenly distributed across the 160mm loop. Turbulence, gusts from the fan starting, or uneven flow pop the film. Fan selection, duct geometry, and PWM ramp profile are all driven by this. |
| Extracting a usable force signal from the wand arm in an outdoor vibrating system | The strain gauge must resolve 50–200 mN of slowly changing force (0.1–1 Hz inflation signal) while rejecting motor vibration (10–100 Hz), wind buffeting, and thermal drift. Signal conditioning, mechanical isolation of the gauge from vibration sources, and firmware filtering all flow from this problem. |

#### Component Choice Architecture

| Component | Dominant Axis | Key Tension | Resolution Direction |
| --- | --- | --- | --- |
| MCU | Cost + firmware complexity | ATtiny / low-end AVR ($0.50) has limited ADC and timers for simultaneous PWM + HX711 polling. ARM Cortex-M0 ($0.80–1.20, e.g., STM32C011 or RP2040) has plenty of peripherals. | ARM Cortex-M0 — the $0.50 premium buys multiple PWM channels, DMA for HX711, and enough flash for the optimization algorithm. |
| Strain gauge + amp | Cost vs. performance | Cheap foil strain gauge ($0.30) + HX711 ($0.50) = $0.80 total, 10–80 Hz, 24-bit. A pre-packaged load cell ($2–5) is easier to integrate but heavier and more expensive. | Foil strain gauge bonded to wand arm + HX711 — lowest cost, lightest, and the wand arm IS the flexure element. Requires factory calibration (zero with no load). |
| Pivot actuator | Firmware complexity vs. cost | Hobby servo ($2) has built-in position control but limited to 180° and fixed speed profile. Geared DC motor ($1.50) + H-bridge ($0.30) needs firmware position control but allows custom velocity profiles. | Geared DC motor + H-bridge — custom velocity profile is critical for not breaking the film. Servo's fixed speed and detent torque are wrong for this application. |
| Fan / blower | Physical constraint vs. performance | Large slow fan (80mm, $1.50) produces gentler airflow but takes more space. Small fast fan (40mm, $0.80) needs a duct/diffuser to slow the air, adding complexity. | Large slow fan — the gentler the airflow, the better for large bubbles. The product is already ~200mm wide for the vat, so an 80mm fan fits. |
| Battery | Cost vs. runtime | 4×AA ($0.50 holder, user-supplied cells) is cheapest but lower energy density. LiPo 2S ($4–6) gives longer runtime but adds charging circuit cost. | 4×AA for V1 — lowest cost, user-replaceable, no charging circuit needed. Revisit LiPo for V2 if runtime is insufficient. |

#### Three Hardest Problems

Film survival during rotation: The soap film must survive the wand arm rotating from horizontal (where it picked up the film) to vertical (where the fan inflates it). This transition takes ~1–2 seconds, during which gravity drains the film and any mechanical vibration tears it. The motor velocity profile, arm stiffness, and pivot smoothness all must be tuned together — and the optimal profile depends on solution chemistry and ambient conditions.

Force-curve interpretation for adaptive control: The strain gauge gives a continuous force signal during inflation. Firmware must classify each cycle's outcome (successful detach, premature pop, no-film, partial inflation) from the force curve shape, then adjust 4+ parameters (fan ramp rate, peak fan speed, dip duration, soak time) to improve the next cycle. The adaptation algorithm must converge quickly (within 5–10 cycles) and handle changing conditions (wind, solution aging, temperature).

Wind compensation at 4 kph: A 4 kph crosswind applies ~10–30 mN of lateral force on a 500mm bubble and distorts the inflation dynamics. The force curve changes shape in wind. Firmware must detect wind presence (asymmetric force signature, faster-than-expected force rise) and compensate — likely by increasing fan speed slightly and shortening blow duration to release the bubble before wind tears it. This interacts with problem #2: the adaptation must distinguish "wind changed" from "solution degraded."

#### Open Calls

| Decision | Options | Deadline |
| --- | --- | --- |
| Wand arm position sensing | (A) Timed open-loop with end-stop switches, (B) Potentiometer on pivot shaft, (C) Encoder on motor | Before schematic — affects firmware complexity and pivot driver circuit |
| Adaptation algorithm | (A) Lookup table with hill-climbing on success rate, (B) PID on force-curve features, (C) Simple gradient descent on parameters | Before firmware architecture — determines MCU flash/RAM needs |
| Vat attachment | (A) Removable snap-in vat with lid for transport, (B) Integrated vat (simpler, but harder to fill/clean) | Before mechanical design — affects enclosure mold and user flow |
| Solution level sensing | (A) Include capacitive sensor on vat wall ($0.10 + firmware), (B) Skip — user checks visually | Before schematic — minor cost but affects BOM and UX |

# Component Arrangement

## Component Inventory

Wand loop — 160mm Ø, flexible (bends flat against vat bottom during dip), attached to wand arm

Wand arm — ≤30mm rigid rod, connects loop to rotating shaft

Rotating shaft — ~180mm span across vat width, axis sits above vat rim +5mm clearance, ~175° rotation range (90° upright to 265° dip), shaft on right side next to air duct protrusion

Open vat — oval, 200mm × 170mm internal, 20mm deep, removable shallow tray

Blower fan — centrifugal/radial, ~40×40×10mm, flat profile, optimized for pressure

Air duct (L-bent horn) — vertical segment from fan, 90° bend to horizontal exit; round outlet Ø40mm, concentric with and flush to upright loop

Geared DC motor — small (~15×10×8mm) with high-ratio gear train (~100:1), drives shaft rotation

Strain gauge — ~10×5mm foil gauge bonded near shaft bearing/pivot

HX711 ADC — integrated on main PCBA

Main PCBA — ~50×35mm; MCU (Cortex-M0), HX711, MOSFET fan driver, H-bridge pivot driver, voltage regulator

Battery holder (4×AA) — ~58×58×15mm (2×2 flat), near base level inside U-rim tall side, aligned with long edge

User controls — power button + mode selection, style TBD

Status LEDs — 3–5× 3mm LEDs

Enclosure: Base — flat plate, footprint ~215×206mm, with 4× leveling rubber feet

Enclosure: Inverse-U rim — asymmetric: tall on one long side (~80mm+ above vat rim, houses duct + PCBA + battery; motor + fan mounted on rim, wired to PCBA), minimal elsewhere (~15mm, holds shaft bearings)

Enclosure: Vat — shallow oval tray (200×170mm internal, 20mm deep), sits on top of base within U-rim

## Selected Views

Side cross-section (looking along the 200mm long axis): Loop and wand arm appear edge-on as a vertical line in blow position. Shows the tapered protrusion on the right side (wide at base, narrows to duct exit + 10mm radial clearance). Shaft pivot on the right, arm rotates 175° from 90° (upright, blow) to 265° (down-left, dip into vat). Reveals the asymmetric U-rim profile and internal component stacking (battery near base, PCBA above, motor at shaft height, fan near top).

Front view (looking along the airflow axis, into the loop): Shows the concentric alignment of the Ø40mm duct exit inside the 160mm loop. Shows the shaft spanning the vat width with bearings at both ends.

## Arrangement Diagram

## Arrangement Rationale

| Placement | Reason |
| --- | --- |
| Vat on top, open | Gravity keeps solution in place; easy to fill and inspect level; loop dips down into it naturally |
| Shaft axis just above vat rim | Minimal clearance allows the arm to swing freely between dip (into vat) and blow (above vat) positions without the shaft interfering with the vat walls |
| Loop above vat in blow position | Bubbles inflate upward and outward, away from the machine — no interference with structure |
| Flexible loop | Bends flat against vat bottom during dip for maximum film coverage and thick coating; springs back to circular when upright |
| Battery near base, aligned with long edge | Low center of gravity for stability; long-edge alignment fits 2×2 AA holder within rim depth |
| PCBA aligned with long edge above battery | Short wire runs to battery below and connectors to motor/fan mounted on rim above |
| Motor + fan mounted on rim, wired to PCBA | Mechanical components fixed to structural rim; electrical connection via JST cables to PCBA — separates mechanical mounting from electronics |
| Tapered protrusion (tall side) | Tapers from full base width at bottom to 60mm (duct Ø40mm + 10mm clearance each side) at duct exit — houses battery, PCBA, motor, and fan while minimizing visual bulk at the top |
| L-bent air horn | Allows a flat centrifugal fan (horizontal) to deliver air horizontally through the vertical loop; the 90° bend fits within the U-rim structure; round exit disperses air evenly |
| Duct exit concentric with loop | Air passes through the center of the soap film, inflating it symmetrically into a sphere |
| Motor on shaft end inside U-rim | Protected from splashes; gear train provides slow, smooth rotation; shaft transmits torque across vat width |
| Strain gauge near shaft bearing | Picks up torsional/bending force from wand arm during inflation; close to pivot = maximum signal; inside dry zone |

## Spatial Constraints Identified

Shaft bearing alignment: Both bearings must be coaxial across ~180mm span. U-rim mounting points must maintain alignment after assembly.

Duct exit alignment: The Ø40mm duct exit must be centered on the loop center when the arm is in blow position. Positional tolerance ±5mm.

Vat clearance for loop rotation: The vat's 200mm long axis accommodates 5mm + 30mm (arm) + 160mm (loop) + 5mm = 200mm exactly. No excess — tight fit ensures compact product.

Wet/dry boundary: The shaft penetrates from the wet zone (vat) into the dry zone (U-rim). Shaft seal or labyrinth required at both bearing points.

Fan intake: The centrifugal fan needs an air intake on the U-rim exterior. Must be positioned to avoid drawing in soap spray.

## Key Dimensions

| Dimension | Value |
| --- | --- |
| Overall footprint | ~215 × 206mm |
| Overall height (to loop top) | ~250mm (base to top of upright loop) |
| U-rim tall side height | ~80mm above vat rim |
| U-rim minimal height | ~15mm above vat rim |
| Vat internal | 200 × 170mm oval, 20mm deep |
| Loop diameter | 160mm |
| Wand arm length | 30mm |
| Shaft span | ~180mm |
| Base width (short axis) | ~206mm (vat 176mm + motor 15mm + clearances 15mm) |
| Duct exit diameter | 40mm |
| Rubber leveling feet | 4×, adjustable or press-fit, on base corners |

# System Description

| Field | Value |
| --- | --- |
| Version | 0.1 |
| Date | 2026-02-25 |
| Author | Electrum workflow |
| Status | Draft |
| Related docs | explore_notes.md, high_level_design.md, component_arrangement.md |

## 1. Product Vision and Context

Product statement:

For families, performers, and outdoor event organizers, Bubbler is a battery-powered automated soap bubble machine that produces large bubbles (up to ~500mm diameter) at a steady pace, using force-sensing feedback to auto-optimize bubble size and stability in light wind. Unlike cheap rotating-disk bubble machines that produce only small bubbles unreliably, Bubbler produces large, stable bubbles consistently — even in crosswinds up to 4 kph.

Problem:

Large soap bubbles captivate audiences but are difficult to produce reliably by hand — they require precise airflow speed, soap film thickness, and timing. Existing consumer bubble machines use spinning disks with fixed fan speed, producing small (20–50mm) bubbles with high pop rates and no adaptation. A machine that can produce large bubbles at a good pace and adapt to conditions (wind, solution age, temperature) fills the gap between cheap toys and professional stage equipment.

Deployment context:

Environment: Outdoor — parks, backyards, event venues, street performance

Setting: Portable, placed on a table or the ground

User type: Consumer — families, entertainers, event planners

Installation: None — fill vat with solution, insert batteries, press power

Expected lifespan: 2–3 years (outdoor toy with wet/dry cycles, mechanical wear on pivot)

## 2. User Scenarios

### Scenario 1: Backyard Party

Persona: Sarah, parent hosting a children's birthday party outdoors.

Situation: Sets up Bubbler on a patio table 30 minutes before guests arrive. Light breeze (~3 kph).

Action: Fills the vat with soap solution (premixed concentrate + water). Presses power. Selects continuous mode. Bubbler begins cycling: dip → rotate up → inflate → release. First few cycles, the firmware calibrates — one or two pops, then it adapts fan ramp and dip duration. Within 5–6 cycles, large bubbles (~400mm) are releasing steadily every 8–12 seconds.

Outcome: A steady stream of large bubbles drifting across the yard. The machine runs unattended for 2+ hours until the solution runs low. LEDs blink amber when solution is low.

### Scenario 2: Street Performer

Persona: Diego, a street performer who uses giant bubbles as part of his act.

Situation: Sets up Bubbler on a small folding table in a pedestrian plaza. Wind is variable, 2–5 kph with gusts.

Action: Uses single-bubble mode — each cycle produces one bubble, waits for a button press to start the next. Between cycles, Diego adjusts his position relative to the wind. The firmware detects windier conditions via asymmetric force curves and automatically increases fan speed slightly and shortens blow duration.

Outcome: Large bubbles (~300–500mm depending on wind) released on cue. Diego controls timing; the machine handles the physics.

### Scenario 3: First Use — Unboxing

Persona: A buyer who just received the product.

Situation: Opens box, reads quick-start card.

Action: Inserts 4×AA batteries. Mixes soap solution using included concentrate + water, pours into vat. Presses power button. Green LED lights (powered). Presses mode button to select continuous mode. The wand arm dips into the vat, soaks for ~2 seconds, rotates up. Fan ramps gently. First bubble may pop (firmware starts with conservative default parameters). By cycle 3–5, bubbles are forming and releasing.

Outcome: Working bubbles within 2 minutes of setup. No app needed, no pairing, no configuration.

### Scenario 4: Error — Solution Runs Out

Persona: Sarah, mid-party.

Situation: Solution level drops below the loop's reach. The wand dips but picks up no film.

Action: Firmware detects no-film condition via the force curve — no force buildup during blow phase (flat signal). After 3 consecutive no-film cycles, Bubbler pauses and blinks amber LED. If capacitive solution sensor is included, it triggers immediately on low level.

Outcome: Machine stops cycling to avoid running the fan with no film (wasting battery). User sees amber LED, refills vat, presses button to resume.

### Scenario 5: Error — Wind Too Strong

Persona: Diego, wind picks up to 8+ kph.

Situation: Bubbles pop during inflation on every cycle. Force curve shows rapid rise then immediate drop (pop signature).

Action: Firmware detects 5+ consecutive pops. Increases fan speed to maximum compensation. If still popping, enters "wind pause" — stops cycling, blinks red LED twice every 5 seconds. Resumes automatically when user presses button (to try again after repositioning) or after a 2-minute timeout.

Outcome: Machine doesn't waste solution cycling in unworkable wind. User repositions or waits for a lull.

## 3. System Architecture

Architecture narrative:

Bubbler is a standalone, battery-powered electromechanical device with no wireless connectivity. The system consists of a mechanical subsystem (wand arm with flexible loop, rotating shaft, open vat, centrifugal blower with L-bent air duct) driven by a firmware control loop running on an ARM Cortex-M0 MCU.

The core signal chain: strain gauge on the wand arm pivot → HX711 24-bit ADC → MCU firmware. The firmware samples the force curve at 10–50 Hz during each blow cycle, analyzes the shape (slope = inflation rate, peak = bubble size proxy, sudden drop = pop, smooth peak → gradual decline = clean detach), classifies the outcome, and adjusts parameters for the next cycle. The controlled parameters are: fan PWM ramp profile, peak fan speed, dip duration (how long the loop soaks), and pause-before-blow (settling time after rotation).

Power comes from 4×AA batteries (6V nominal) through a 3.3V regulator for the MCU and HX711, with the motors driven from the battery rail via MOSFET (fan) and H-bridge (pivot motor). The user interface is minimal: a power button, a mode button (continuous / single / demo), and 3–5 status LEDs.

There is no app, no cloud, no wireless. All intelligence is on-device. The firmware's optimization loop runs autonomously — the user fills the vat, presses power, and walks away.

## 4. Subsystem Descriptions

One subsection per subsystem lead in team.md: 4.1 Electrical, 4.2 Firmware, 4.5 Mechanical / industrial design. 4.3 and 4.4 are kept to record that V1 has no app and no cloud. Each subsection ends with its Handoff block.

### 4.1 Hardware Subsystem

MCU / SoC:

Part: STM32C011F4 (or STM32C011F6 if more flash needed)

Selection rationale: Cheapest ARM Cortex-M0+ with sufficient peripherals. No wireless needed (standalone product), so BLE-capable MCUs (nRF52, ESP32) are wasted cost. The STM32C011 provides: 2× hardware timers for PWM (fan + motor), SPI or bit-banged interface for HX711, ADC channel for battery voltage, and GPIOs for buttons/LEDs. At $0.70–0.90 in volume, it's the right price for a sub-$50 product.

Key specs: 48 MHz Cortex-M0+, 16–32 KB flash, 6 KB RAM, 12-bit ADC, 2× 16-bit timers with PWM, SPI, UART, up to 15 GPIO

Sensors:

| Sensor | Measures | Interface | Sample Rate | Key Spec |
| --- | --- | --- | --- | --- |
| Foil strain gauge (120Ω, bonded near shaft pivot) | Force on wand arm during bubble inflation | HX711 ADC (24-bit, differential) | 10–80 Hz (HX711 selectable) | Sensitivity: ~2 mV/V at full range. Resolves ~5 mN force changes. Measures 50–200 mN total range during inflation. |
| Battery voltage divider (2× resistors) | Battery voltage for fuel gauge | MCU ADC channel | 1 Hz | Detects low battery (<4.2V for 4×AA), triggers amber LED and reduced-power mode |
| Capacitive solution level (optional) | Soap solution present/absent in vat | MCU GPIO (capacitive touch peripheral or RC time constant) | 1 Hz | Binary threshold: solution present above loop level, or low. Simple and cheap (~$0.10 for electrode + firmware). |

Actuators:

| Actuator | Function | Interface | Key Spec |
| --- | --- | --- | --- |
| Centrifugal blower fan (~40×40×10mm) | Generate directed airflow through L-bent duct to inflate soap film | MCU PWM → N-channel MOSFET | Variable speed 0–100% via PWM. Target air velocity at duct exit: 0.5–2 m/s. Pressure optimized (centrifugal) for duct losses. 5V or 6V rated. |
| Geared DC motor (~15×10×8mm, ~100:1 ratio) | Rotate shaft ~175° (dip position ↔ blow position) | MCU PWM + DIR → H-bridge (DRV8837 or similar) | Slow, smooth rotation (~1–2 seconds per 100° sweep). Low torque required (loop + arm < 50g). High gear ratio for smoothness and holding torque. |

Physical UI elements:

Buttons: 2× tactile pushbuttons — power on/off, mode select (cycle through: continuous / single / demo)

LEDs: 3–5× 3mm LEDs — green (power on), blue (running/cycling), amber (low solution or low battery), red (error / wind pause). Possibly one RGB LED instead.

Display: None

Other: None

PCB strategy:

Single board, 2-layer FR4, ~50 × 35mm

Components: STM32C011 (TSSOP20 or QFN), HX711 (SOP16), DRV8837 H-bridge (WSON8), N-ch MOSFET for fan, 3.3V LDO (MCP1700 or AP2112K), battery voltage divider, LED drivers (resistors), button pull-ups, strain gauge connector, motor connector, fan connector, battery connector

All through-hole connectors (JST-PH) for wire harness to motors, fan, strain gauge, battery, LEDs, buttons

SWD debug/programming pads (Tag-Connect TC2030 footprint for production, 0.1" header for dev)

Board mounts inside the sealed electronics compartment in the tall side of the U-rim

Handoff

Lead: Electrical / electronics lead

Deliverables at gate: schematic and layout released; strain-gauge signal chain measured on the bench (noise floor against the 5 mN resolution target, with the motor running); EVT power budget measured against §6; FCC Part 15B / CE EMC pre-scan plan

Needs from other domains: PCB outline, mounting bosses and connector exits from Mechanical (§4.5); pin map and PWM frequencies agreed with Firmware (§4.2); gauge bonding location from Mechanical (open question #1)

### 4.2 Firmware Subsystem

Architecture:

OS/framework: Bare-metal (superloop with timer interrupts)

Rationale: The system has only two time-critical tasks: HX711 sampling (10–80 Hz, interrupt-driven) and PWM generation (hardware timer). Everything else (state machine, optimization, LED management) runs in the main loop at ~100 Hz. No RTOS overhead is justified for a $0.80 MCU with 6 KB RAM.

Major modules:

| Module | Responsibility | Inputs | Outputs |
| --- | --- | --- | --- |
| HX711 driver | Bit-bang SPI to read 24-bit force samples from HX711 at configured rate (10 or 80 Hz). Applies tare offset. | HX711 DOUT + SCK pins | Raw force value (grams-force, tared) |
| Force curve analyzer | Buffers force samples during each blow cycle. Extracts features: peak force, slope during inflation, time to peak, drop rate after peak. Classifies outcome: success (clean detach), pop (abrupt mid-rise drop), no-film (flat), partial (low peak). | Buffered force samples | Cycle outcome classification + extracted features |
| Optimization controller | Adjusts parameters for next cycle based on outcome history. Maintains a parameter set: {fan_ramp_rate, fan_peak_speed, dip_duration_ms, soak_pause_ms, blow_duration_ms}. Uses simple hill-climbing: on success, make small random perturbation; on failure, revert last change and try a different direction. Tracks rolling success rate over last 10 cycles. | Cycle outcome + features | Updated parameter set for next cycle |
| Cycle state machine | Sequences the dip-rotate-blow cycle: IDLE → DIP_DOWN → SOAK → ROTATE_UP → SETTLE → BLOW → RELEASE → ROTATE_DOWN → repeat. Manages timing for each phase using current parameter set. | Mode (continuous/single), optimization parameters, button events | Motor commands, fan commands, phase timing |
| Motor controller | Drives pivot motor via H-bridge with soft-start/stop velocity profiles. Uses timed open-loop control (no encoder): run motor at calibrated PWM for calibrated duration to achieve ~175° rotation. End-stop detection via stall current or limit switches (TBD). | Direction + speed commands from state machine | H-bridge PWM + DIR signals |
| Fan controller | Drives blower fan via MOSFET PWM. Implements configurable ramp profile: linear or exponential spin-up from 0 to target speed over configurable duration. Gentle ramp is critical — sudden air blast pops the film. | Fan speed target + ramp parameters from state machine | Fan MOSFET PWM signal |
| Battery monitor | Samples battery voltage at 1 Hz via ADC. Applies low-pass filter. Detects low battery (< 4.2V for 4×AA) and critical battery (< 3.8V). On low: amber LED. On critical: stop cycling, red LED. | ADC reading | Battery state (OK / low / critical) |
| LED manager | Drives LED GPIOs based on system state. Supports solid, blink, and pulse patterns. Priority: red (error) > amber (warning) > blue (running) > green (power). | System state flags | GPIO outputs |
| Solution monitor (optional) | Reads capacitive level sensor. Detects solution absent. After 3 consecutive no-film + no-solution readings, pauses and alerts. | Capacitive sense GPIO | Solution state (OK / low) |

OTA update strategy:

None for V1. Firmware programmed at factory via SWD. Field updates require physical access (Tag-Connect or pogo-pin jig). Acceptable for a consumer toy at this price point.

On-device processing:

Force curve feature extraction: peak detection, slope calculation, drop-rate measurement — all integer math on 24-bit samples

Optimization: hill-climbing on 5 parameters with rolling success-rate tracking. No floating point required. State persisted in flash (last-known-good parameter set survives power cycles).

All processing is local. No connectivity, no external dependencies.

Boot-to-ready: ~500ms (power-on → clock init → peripheral init → tare strain gauge → enter IDLE state → green LED on)

Handoff

Lead: Firmware lead

Deliverables at gate: skeletons for the nine modules above; force-curve classifier validated on recorded prototype curves (success / pop / no-film / partial); hill-climbing convergence shown within 10 cycles on the bench; firmware version struct in flash readable over SWD (gate gap #34)

Needs from other domains: calibrated PWM-to-angle figures and end-stop behaviour from Mechanical; HX711 wiring, pin map and battery thresholds from Electrical; production test sequence agreed with Manufacturing / NPI

### 4.3 Mobile / Companion App Subsystem

Not applicable. Bubbler V1 is fully standalone. No app, no connectivity.

Future V2 option: BLE companion app showing real-time force curves (satisfying to watch), cycle statistics, and target bubble size adjustment. Would require adding an nRF52810 (~$1.50) or ESP32-C3 (~$1.20) as a BLE co-processor, connected to the STM32 via UART. The PCBA has space reserved for this.

Handoff

Lead: none in V1 (no App lead on the team)

Deliverables at gate: none

Needs from other domains: Electrical keeps the UART header and board space reserved for a V2 BLE co-processor

### 4.4 Cloud / Backend Subsystem

Not applicable. No cloud, no accounts, no data collection.

Handoff

Lead: none in V1 (no Cloud / backend lead on the team)

Deliverables at gate: none

Needs from other domains: none

### 4.5 Mechanical and Structural Subsystem

Actuators and drives:

Pivot: geared DC motor ~15×10×8mm, ~100:1 ratio, driving a stainless shaft spanning ~180mm on two bearings; ~175° sweep between the dip end-stop (loop horizontal, submerged) and the blow end-stop (loop vertical, facing the duct); 1–2 s per 100° with soft-start/stop; load < 50 g (arm + loop + film), so torque is not the constraint, smoothness is

Airflow: centrifugal blower ~40×40×10mm, PWM 0–100%, through an L-bent duct with a Ø40mm exit aimed at the vertical loop; target 0.5–2 m/s at the loop, pressure-optimised for duct losses

Wand: 30mm arm with a 160mm loop (spring-wire or coated wire, open question #4); the arm is the flexure element for the strain gauge

Structure:

Three injection-moulded parts, ABS or PP: base (battery compartment, feet, motor mount), tapered U-rim (sealed electronics compartment on the tall side, duct and blower in the protrusion, shaft bearings), removable vat (200 × 170mm oval, 20mm deep, ~500mL, snap-on lid for transport)

Load paths are small (< 1 N dynamic); the design constraint is arm stiffness so flex does not corrupt the gauge signal, and a pivot with no backlash or stick-slip

Footprint 215 × 206mm, height ~250mm to the loop top, 350–450 g without cells; four rubber levelling feet

Simple draft-angle moulds, no undercuts; tooling 8 weeks, the longest item on the schedule

Physical interfaces:

Wet/dry boundary at the shaft: labyrinth seal where the shaft enters the U-rim; strain gauge on the dry side near the bearing, conformal-coated

IPX4 electronics compartment; vat and wand fully wettable

Battery compartment sealed from the vat side; tool-release latch if the product is classified as a toy (open question #6)

Harness exits for motor, blower, end-stop switches and gauge, all JST-PH into the sealed compartment

Control loops:

Position: timed open-loop PWM with two end-stop micro-switches (decision #6); calibrated at the factory

Film: force curve from the gauge at 10–80 Hz drives the cycle timing (dip, soak, rotate, blow) through the firmware optimiser (§4.2); mechanical requirement is a vibration path from motor to gauge below the 5 Hz firmware cutoff's noise budget

Safety: motor stall detection by current or end-stop timeout; fan ramp never steps, always ramps

Handoff

Lead: Mechanical / industrial design lead

Deliverables at gate: enclosure CAD for the three parts ready for tooling quotes; wand, shaft, bearing and seal drawings; duct airflow measured at the loop (0.5–2 m/s across the 160mm opening); gauge mounting location and bonding method decided (open question #1); loop material decided (open question #4)

Needs from other domains: PCBA outline and connector positions from Electrical; motor velocity profile limits from Firmware; mould and bonding-step quotes from Manufacturing / NPI

## 5. Interfaces

### Internal Interfaces (within device)

| Interface | From | To | Protocol | Data | Rate | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Force sensing | Strain gauge (Wheatstone bridge) | HX711 ADC | Analog differential (µV–mV) | Force proportional to wand arm load | Continuous (analog) | 120Ω gauge, 3.3V excitation, ~2 mV/V sensitivity |
| Force data | HX711 DOUT pin | MCU GPIO (bit-banged SPI) | Serial clock + data (proprietary HX711 protocol) | 24-bit signed force reading | 10 Hz or 80 Hz (selectable via HX711 RATE pin) | MCU generates SCK, reads DOUT. Data ready signaled by DOUT going low. |
| Fan speed | MCU PWM timer | N-ch MOSFET gate | 25 kHz PWM, 0–100% duty | Fan speed command | Updated per cycle phase (ramp profile) | MOSFET: IRLML6344 or similar logic-level N-ch. Fan on battery rail (6V). |
| Motor direction + speed | MCU GPIO × 2 | DRV8837 H-bridge (IN1, IN2) | Logic-level GPIO: IN1=H/IN2=L → forward, IN1=L/IN2=H → reverse, both L → brake/coast | Motor direction + speed | State changes during cycle (dip down / rotate up) | DRV8837: 1.8A peak, built-in shoot-through protection. Motor on battery rail. |
| Battery voltage | Resistor divider (100kΩ + 100kΩ) | MCU ADC | Analog, 0–3.3V (half of battery voltage) | Battery state | 1 Hz | Divide by 2: 6V battery → 3V at ADC. 12-bit resolution = ~1.5 mV/step. |
| Buttons | 2× tactile switches | MCU GPIO (internal pull-up) | Active-low digital | Button press/release | Event-driven (interrupt + debounce) | 20ms debounce in firmware |
| LEDs | MCU GPIO × 3–5 | LEDs via current-limiting resistors | Logic-level GPIO, sink or source | LED state | State-driven | 10–20mA per LED, 150Ω–330Ω resistors |
| Solution level (optional) | Capacitive electrode on vat wall | MCU GPIO | RC time constant measurement | Solution present / absent | 1 Hz | Electrode is a copper trace on a flex PCB adhered to vat exterior. Firmware measures charge time. |

### External Interfaces (device to outside world)

| Interface | From | To | Protocol | Data | Rate | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Battery | 4×AA cells in holder | Device power rail | Direct DC, 4.5–6.5V (1.0–1.6V per cell × 4) | Power | — | User-replaceable. Alkaline or NiMH. |

### Physical Connectors

| Connector | Purpose | Type | Notes |
| --- | --- | --- | --- |
| Battery holder (4×AA) | Power | 2×2 AA holder with snap-fit leads + JST-PH 2-pin to PCBA | User-accessible compartment with thumb-screw or snap lid |
| Strain gauge | Force signal to HX711 | JST-SH 4-pin (E+, E-, S+, S-) | Shielded cable, ~100mm, routed through U-rim from gauge near shaft to PCBA |
| Motor | Pivot drive | JST-PH 2-pin | ~80mm wire, internal routing |
| Fan | Blower drive | JST-PH 2-pin | ~60mm wire, internal routing |
| SWD debug/programming | Factory firmware load + debug | Tag-Connect TC2030-CTX (6-pin, no-legs) | Pogo-pin jig, not user-accessible. Pads on PCBA. |
| Vat contact (optional) | Capacitive level sense | Flex PCB electrode adhered to vat exterior | No connector — direct solder or ZIF to main PCBA |

## 6. Power Architecture

Power source: 4×AA batteries in a 2×2 flat holder. 6V nominal (alkaline), 4.8V nominal (NiMH). No charging circuit — user-replaceable cells.

Power rails:

| Rail | Voltage | Source | Load |
| --- | --- | --- | --- |
| V_BAT | 4.5–6.5V (varies with chemistry and discharge) | Battery holder direct | Fan MOSFET, H-bridge motor driver |
| V_LOGIC | 3.3V regulated | LDO (MCP1700-33 or AP2112K-3.3) from V_BAT | MCU, HX711, LEDs, button pull-ups |

Power budget (continuous mode, typical cycle):

| Consumer | Active Current | Duty Cycle | Average Power |
| --- | --- | --- | --- |
| Blower fan (peak) | 300–500mA @ 6V | ~40% (blow phase only) | ~0.9W average |
| Pivot motor (moving) | 150–250mA @ 6V | ~20% (rotate up + rotate down) | ~0.25W average |
| MCU + HX711 | 15mA @ 3.3V | 100% | ~0.05W |
| LEDs (status) | 10–20mA @ 3.3V | 100% (solid or blink) | ~0.05W |
| LDO quiescent + divider | ~5mA | 100% | ~0.03W |
| Total average |  |  | ~1.3W |

Battery life estimate:

4×AA alkaline: ~9 Wh usable capacity → ~7 hours continuous operation

4×AA NiMH (2000mAh): ~10 Wh → ~8 hours continuous operation

Target ≥2 hours: met with wide margin

Low-battery behavior:

V_BAT < 4.2V: amber LED, firmware reduces fan peak speed by 20% to extend runtime

V_BAT < 3.8V: red LED, cycling stops, MCU enters low-power sleep (~10µA). User must replace batteries.

Power sequencing: None required. LDO output rises with V_BAT. MCU brown-out detector (BOR) holds reset until V_LOGIC ≥ 2.7V.

## 7. Connectivity

Not applicable. Bubbler V1 has no wireless or wired data connectivity.

No BLE, Wi-Fi, cellular, USB data, or any other communication interface. The product is fully standalone. All intelligence runs on-device.

V2 consideration: Space reserved on PCBA for a BLE co-processor (nRF52810 or ESP32-C3) connected via UART. This would enable a companion app for force-curve visualization and parameter tuning. Not in V1 scope.

## 8. Key Decisions

| # | Decision | Options Considered | Choice | Rationale |
| --- | --- | --- | --- | --- |
| 1 | Force sensing method | (A) IR break-beam across loop, (B) Motor current sensing, (C) Pressure sensor in duct, (D) Strain gauge on wand arm | (D) Strain gauge | Directly measures the inflation force (50–200mN) at the wand arm. Cheapest ($0.80 for gauge + HX711). Lightest. The wand arm itself is the flexure element — no added mechanical complexity. Other options either measure indirect proxies or add cost. |
| 2 | MCU selection | (A) ATtiny/AVR ($0.50), (B) STM32C011 Cortex-M0+ ($0.80), (C) RP2040 ($0.80), (D) ESP32-C3 ($1.20) | (B) STM32C011 | The $0.30 premium over ATtiny buys multiple PWM channels, enough flash for the optimization algorithm, and a proper debug interface. No wireless needed, so ESP32 is wasted. RP2040 is dual-core overkill. STM32 ecosystem has mature bare-metal HAL. |
| 3 | Pivot actuator | (A) Hobby servo ($2), (B) Geared DC motor + H-bridge ($1.80) | (B) Geared DC motor | Custom velocity profile is critical for film survival during rotation. Servo's fixed speed profile and detent torque are wrong for this application. The geared DC motor allows firmware-controlled soft-start/stop ramps. |
| 4 | Fan type | (A) Axial fan 60–80mm, (B) Centrifugal blower 40×40mm | (B) Centrifugal blower | Centrifugal fans produce higher static pressure at low flow — needed to push air through the L-bent duct with minimal turbulence. Flat 40×40×10mm profile fits inside the tapered protrusion. Axial fans are larger and produce more turbulence at the exit. |
| 5 | Battery type | (A) 4×AA user-replaceable, (B) LiPo 2S with charging circuit | (A) 4×AA | Lowest BOM cost ($0.50 for holder vs. $4–6 for LiPo + charging IC + protection). No charging circuit complexity. User-replaceable cells mean no returns for dead batteries. Revisit LiPo for V2 if runtime or form factor demands it. |
| 6 | Wand position sensing | (A) Timed open-loop + end-stop switches, (B) Potentiometer on shaft, (C) Encoder on motor | (A) Timed open-loop + end-stops | Simplest and cheapest. The motor runs at a calibrated PWM for a calibrated duration. Two micro-switches at the end-of-travel positions (blow and dip) provide absolute reference. No analog read or encoder decoding needed. |
| 7 | Enclosure construction | (A) Unified shell, (B) Base + U-rim + removable vat (3-part) | (B) 3-part | Vat must be removable for filling, cleaning, and transport. Tapered U-rim provides structural support and houses all electronics in a sealed compartment. Simple injection molds — no undercuts. |
| 8 | Adaptation algorithm | (A) Lookup table + hill-climbing, (B) PID on force features, (C) Gradient descent | (A) Hill-climbing | Simplest to implement in 16KB flash with integer math. Five parameters, binary outcome (success/fail) — hill-climbing converges in 5–10 cycles. PID requires continuous error signal; gradient descent needs more RAM for state. Hill-climbing's random perturbation also provides implicit exploration of the parameter space. |

## 9. Constraints

### Regulatory

FCC Part 15 Class B (US): Unintentional radiator — the MCU clock (48 MHz) and PWM switching (25 kHz) are potential EMI sources. 2-layer PCB with ground plane, short traces, and decoupling caps should meet limits. No formal pre-scan expected to be needed for this low-frequency design.

CE (EU): EMC Directive (EN 55032 Class B), Low Voltage Directive does not apply (< 50V DC). RoHS compliance for all components.

No radio: No FCC ID or CE RED required — no intentional radiator.

Toy safety (if marketed to children): ASTM F963 (US), EN 71 (EU) — small parts, battery compartment security, sharp edges. The battery compartment needs a screw or tool-release mechanism if targeting ages < 14.

### BOM Cost

| Category | Buy / Make | Target | Lead time | MOQ | Notes |
| --- | --- | --- | --- | --- | --- |
| MCU + passives | Buy | $1.50 | 8–12 wk | 3k reel | STM32C011 + decoupling + crystal + resistors. Pin-compatible STM32C011F6 as alternate |
| HX711 + strain gauge | Buy | $0.80 | 2–4 wk | 1k | Gauge $0.30 + HX711 $0.50. Several HX711 sources |
| Motor + H-bridge | Buy | $1.80 | 6–8 wk | 5k (motor) | Geared DC $1.50 + DRV8837 $0.30. Motor is the longest-lead mechanical part |
| Fan + MOSFET | Buy | $1.10 | 4–6 wk | 2k (fan) | Centrifugal blower $0.80 + IRLML6344 $0.30 |
| LDO + power | Buy | $0.40 | 2–4 wk | 3k reel | MCP1700 + battery holder |
| PCB (2-layer, panelized) | Make | $0.60 | 4–5 wk (see below) | Per panel | 50×35mm, 10k+ volume |
| Connectors + passives | Buy | $0.80 | 2–4 wk | Full reel | JST-PH × 4, JST-SH × 1, resistors, caps |
| Enclosure (3-part injection mold) | Make | $3.50 | 10–12 wk (see below) | 1k shot run | Base + U-rim + vat, ABS or PP |
| Wand arm + loop + shaft | Make | $1.50 | 4 wk (see below) | 2k | Stainless rod, spring wire loop, bearings × 2 |
| Buttons + LEDs | Buy | $0.30 | 1–2 wk | Full reel | 2× tactile + 4× LED + resistors |
| Misc (rubber feet, labels, screws) | Buy | $0.50 | 1–2 wk | 1k |  |
| Total BOM |  | ~$12.80 | Critical path: enclosure, 10–12 wk |  | Target < $15 for sub-$50 retail at 3.5× markup. Lead times and MOQs are estimates; confirm with quotes |

### Mechanical

Overall footprint: 215 × 206mm

Overall height: ~250mm (base to top of upright loop)

Weight (without batteries): ~350–450g estimated

Weight (with 4×AA alkaline): ~450–550g

Vat capacity: ~500mL (200 × 170mm oval, 20mm deep)

### Environmental

Operating temperature: 5–40°C (outdoor use, temperate climates)

Storage temperature: -20–60°C

Water resistance: IPX4 for electronics compartment (splash-proof). Vat and wand arm are wet by design.

UV resistance: Enclosure material must be UV-stabilized (outdoor use)

### Manufactured Part Lead Times

| Part | Process | Tooling (weeks) | First articles and approval (weeks) | Production run (weeks) | Total (weeks) | Basis |
| --- | --- | --- | --- | --- | --- | --- |
| Enclosure (base, U-rim, vat) | Injection molding, 3 single-cavity molds; T1 samples, one round of fixes | 6–8 | 2 | 2 | 10–12 | Own estimate |
| Wand arm + loop + shaft | Wire forming (jig), rod cut and turned | 1 | 1 | 2 | 4 | Own estimate |
| PCBA | PCB fab + SMT assembly (stencil, programming fixture) | 1 | 1 | 2–3 | 4–5 | Own estimate |

### Assembly and Inspection Times

| Stage | Assembly time per unit | Inspection / test time per unit | Inspection method | Notes |
| --- | --- | --- | --- | --- |
| PCBA | 3 min (at the board assembler) | 2 min | AOI, then firmware flash and board test via Tag-Connect | Hand-soldered JST connectors are the slow step |
| Drive module (motor, gearbox, shaft, bearings, wand arm, loop) | 4 min | 1 min | Spin test, loop alignment gauge |  |
| Fan module (blower in duct) | 2 min | 0.5 min | Run test, airflow check by hand |  |
| Final assembly | 8 min | 3 min | Functional test (wand cycle, fan, strain-gauge tare), cosmetic check; IPX4 spray test on a sample per lot | Bottleneck stage |
| Total per unit | 17 min | 6.5 min |  | Estimates, before line balancing |

### Schedule

Week 0 is the start of detailed design, right after the gate review. Durations are estimates.

| # | Task | Start (week) | Duration (weeks) | Depends on | Owner |
| --- | --- | --- | --- | --- | --- |
| 1 | Schematic and PCB layout | 0 | 4 |  | EE |
| 2 | Enclosure CAD, ready for tooling | 0 | 5 |  | ME |
| 3 | Firmware: control loop and force sensing | 0 | 10 |  | FW |
| 4 | EVT PCB fab and assembly | 4 | 2 | 1 | EE |
| 5 | EVT build and test (3D-printed enclosure) | 6 | 4 | 2, 4 | All |
| 6 | Enclosure tooling | 10 | 8 | 5 | ME + molder |
| 7 | Enclosure first articles (T1) and approval | 18 | 2 | 6 | ME |
| 8 | Geared motor order (purchased) | 10 | 8 | 5 | Ops |
| 9 | DVT PCBA run | 10 | 5 | 5 | EE |
| 10 | Wand assembly jig and samples | 10 | 2 | 5 | ME |
| 11 | DVT build and test | 20 | 4 | 3, 7, 9, 10 | All |
| 12 | FCC / CE testing | 24 | 4 | 11 | EE |
| 13 | PVT pilot run | 24 | 3 | 8, 11 | Ops |
| 14 | Mass production, first lot | 28 | 2 | 12, 13 | Ops |

Critical path: schematic → EVT → enclosure tooling and first articles → DVT → FCC/CE testing → first lot, 30 weeks. Enclosure tooling alone is 8 of them.

Hard deadlines: holiday retail window; first lot must ship by week 30 to reach shelves for the season

Certification timeline: FCC Part 15B and CE EMC testing, 4 weeks including one retest

## 10. Open Questions and Risks

### Open Questions

| # | Question | Impact | Deadline |
| --- | --- | --- | --- |
| 1 | Exact strain gauge placement — bonded to shaft vs. bonded to arm near pivot? | Affects signal strength, noise coupling, and wet/dry boundary design | Before mechanical prototype |
| 2 | End-stop switch type — micro-switch vs. Hall sensor + magnet? | Affects cost ($0.10 vs. $0.40), reliability, and waterproofing at shaft seal | Before schematic |
| 3 | Capacitive solution level sensor — include in V1 or skip? | $0.10 BOM impact; improves UX (immediate low-solution alert vs. 3-cycle detection delay) | Before schematic |
| 4 | Loop material — spring steel wire vs. nylon-coated wire vs. silicone tube? | Affects film pickup, flexibility, durability, and soap compatibility | Before mechanical prototype |
| 5 | Soap solution formula — include premixed concentrate or specify a recipe? | Affects packaging, cost, and bubble quality consistency | Before production |
| 6 | Toy safety classification — is this a toy (ASTM F963 / EN 71) or a general consumer product? | Determines battery compartment design, labeling, and testing requirements | Before industrial design freeze |

### Technical Risks

| # | Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | Film breaks during rotation — motor vibration or jerk tears the soap film before reaching blow position | Medium | High — no bubbles produced | Soft-start/stop motor profiles, high gear ratio for smooth rotation, vibration-damped shaft bearings. Prototype testing with different velocity profiles. |
| 2 | Force signal too noisy for reliable classification — motor vibration and wind couple into strain gauge | Medium | High — optimization loop can't distinguish outcomes | Low-pass filtering in firmware (cutoff ~5 Hz), mechanical isolation of gauge from motor, differential measurement to reject common-mode noise. If insufficient, switch to 80 Hz sampling + digital bandpass. |
| 3 | Hill-climbing gets stuck in local optimum — parameters converge to a mediocre setting | Low | Medium — bubbles work but aren't optimized | Add periodic random restarts (every 50 cycles, try a random parameter set). Store best-known parameter set in flash as fallback. |
| 4 | Centrifugal fan doesn't produce enough pressure through L-bent duct | Low | High — inadequate airflow, no inflation | Size the duct generously (Ø40mm exit), minimize bends, use a higher-pressure blower if needed. Prototype test with different fan models. |
| 5 | Soap solution degrades the strain gauge bond over time | Medium | Medium — sensor drift or failure after months | Conformal coat the gauge. Place it in the dry zone (inside U-rim, near bearing). Use labyrinth seal on shaft penetration. |
| 6 | Enclosure mold cost exceeds budget — tapered protrusion adds tooling complexity | Low | Medium — higher unit cost or redesign | Tapered protrusion is a simple draft-angle surface — standard for injection molding. Three-part enclosure avoids undercuts. Get tooling quotes early. |

# Gate Checklist

| Field | Value |
| --- | --- |
| Date | 2026-02-25 |
| System Description Version | 0.1 |
| Reviewer | Auto |
| Result | PASS with notes — 3 minor gaps, all N/A items justified. Ready for next phase. |

## Vision and Context

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 1 | Product statement is a single clear sentence | PASS | §1 — "Bubbler is a battery-powered automated soap bubble machine that produces large bubbles…" |
| 2 | Problem being solved is stated explicitly | PASS | §1 — existing machines produce only small bubbles unreliably with no adaptation |
| 3 | [HW↔SW] HW vs. SW capabilities stated | PASS | §3 — mechanical subsystem (wand, fan, vat) is HW; optimization loop, force-curve analysis, state machine are FW |
| 4 | [HW↔SW] Software value on top of hardware is clear | PASS | §3 — firmware adds adaptive optimization (hill-climbing on force curves), not just fixed-speed operation |
| 5 | Deployment environment defined | PASS | §1 — outdoor, parks/backyards/events, portable, table or ground |
| 6 | Expected product lifespan stated | PASS | §1 — 2–3 years |

## User Scenarios

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 7 | At least 3 concrete scenarios | PASS | 5 scenarios: backyard party, street performer, unboxing, solution out, wind too strong |
| 8 | [HW↔SW] Each scenario traces through full stack | PASS | Scenario 1: dip → rotate → strain gauge → firmware calibrates → fan ramps → bubble releases. No app/cloud stack (standalone). |
| 9 | At least 1 error/edge-case scenario | PASS | Scenarios 4 (no solution) and 5 (wind too strong) |
| 10 | [HW↔SW] First-time experience described end-to-end | PASS | Scenario 3 — unboxing → batteries → solution → power → first bubbles in 2 minutes |
| 11 | Most common interaction identified | PASS | Scenario 1 — fill, press power, walk away |
| 12 | [HW↔SW] Offline/degraded scenarios covered | N/A | No connectivity — device is always "offline." Degraded modes covered via low-battery and wind-pause behaviors. |

## System Architecture

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 13 | Block diagram covers all major subsystems | PASS | Block diagram includes mechanical domain (vat, wand, fan, motor) and electronic domain (MCU, HX711, drivers, power, UI) |
| 14 | Every subsystem in diagram has a description | PASS | §4.1 (HW), §4.2 (FW) — all blocks described |
| 15 | [HW↔SW] Every HW↔SW arrow specifies protocol, data, direction | PASS | §5 interface table — HX711 serial protocol, PWM frequencies, GPIO directions all specified |
| 16 | Data flows identified | PASS | §5 — 8 internal interfaces with protocol, data, rate |
| 17 | Trust/security boundaries marked | N/A | No connectivity, no external data exchange. Battery-powered toy with no attack surface beyond physical access. |
| 18 | Architecture narrative explains "why" | PASS | §3 — explains force-sensing approach, why standalone, why no RTOS |
| 19 | [HW↔SW] Processing location is clear | PASS | §4.2 — "All processing is local. No connectivity, no external dependencies." |
| 20 | Fundamental HW problems identified | PASS | high_level_design.md — film survival during rotation, gentle airflow, force signal extraction |
| 21 | Resolution paths stated | PASS | Each problem has resolution direction (velocity profiles, duct geometry, filtering + mechanical isolation) |

## Subsystem Descriptions — Hardware

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 22 | MCU selected with rationale | PASS | §4.1 — STM32C011F4, cheapest Cortex-M0+ with sufficient peripherals, $0.70–0.90 |
| 23 | Dominant tradeoff axis per component identified | PASS | high_level_design.md component choice table covers MCU, strain gauge, pivot actuator, fan, battery |
| 24 | Tradeoff conflicts surfaced | PASS | §8 — 8 decisions with options and rationale |
| 25 | All sensors listed with interface, rate, specs | PASS | §4.1 sensor table — strain gauge, battery divider, capacitive level (optional) |
| 26 | Actuators and physical UI listed | PASS | §4.1 — blower fan, geared DC motor, 2 buttons, 3–5 LEDs |
| 27 | PCB strategy described | PASS | §4.1 — single board, 2-layer FR4, 50×35mm, component list, connector types |
| 28 | [HW↔SW] Test points and debug interfaces documented | PASS | §4.1 — SWD debug pads (Tag-Connect TC2030), 0.1" header for dev |

## Subsystem Descriptions — Firmware

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 29 | OS/framework chosen with rationale | PASS | §4.2 — bare-metal superloop, justified by simple task structure and 6KB RAM constraint |
| 30 | Major modules listed with responsibilities | PASS | §4.2 — 9 modules with I/O table |
| 31 | [HW↔SW] HAL boundaries defined | PASS | Implicit — STM32 HAL for GPIO/PWM/ADC, bit-bang SPI for HX711. Application modules use HAL functions. |
| 32 | [HW↔SW] OTA update strategy defined | PASS | §4.2 — "None for V1. Firmware programmed at factory via SWD." Justified for consumer toy at this price point. |
| 33 | [HW↔SW] On-device vs. cloud processing boundary | PASS | §4.2 — all processing local, no data leaves device |
| 34 | [HW↔SW] FW versioning scheme defined | GAP | Not explicitly addressed. Minor for V1 (no OTA, no app), but should define a version byte in flash for factory test/traceability. |

## Subsystem Descriptions — Companion App

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 35–40 | All app items | N/A | §4.3 — "Not applicable." No app in V1. V2 BLE companion noted as future option. |

## Subsystem Descriptions — Cloud / Backend

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 41–44 | All cloud items | N/A | §4.4 — "Not applicable. No cloud, no accounts, no data collection." |

## Interfaces

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 45 | Every internal bus/connection listed | PASS | §5 — 8 internal interfaces (force sensing, force data, fan PWM, motor H-bridge, battery ADC, buttons, LEDs, solution level) |
| 46 | Every external interface listed | PASS | §5 — battery (only external interface) |
| 47 | Physical connectors documented | PASS | §5 — 6 connectors (battery, strain gauge, motor, fan, SWD, vat electrode) |
| 48 | No subsystem is an island | PASS | All blocks connected via interface table |
| 49 | Protocol specified for each interface | PASS | §5 — PWM frequency, bit-bang SPI protocol, analog specs, GPIO polarity |
| 50 | [HW↔SW] HW↔FW interfaces specify signal-level details | PASS | §5 — voltage divider values, MOSFET part, H-bridge part, debounce timing, current-limiting resistor values |
| 51 | [HW↔SW] FW↔App interfaces specified | N/A | No app |
| 52 | [HW↔SW] App↔Cloud interfaces specified | N/A | No cloud |
| 53 | [HW↔SW] Data format transformations documented | PASS | §4.2 — raw 24-bit HX711 → tared grams-force → feature extraction (peak, slope, drop rate) → outcome classification → parameter adjustment |

## Power Architecture

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 54 | Power source and capacity specified | PASS | §6 — 4×AA, 6V nominal, ~9 Wh alkaline / ~10 Wh NiMH |
| 55 | Power states defined with transition triggers | PASS | §6 — low-battery thresholds (4.2V → reduced power, 3.8V → shutdown), BOR at 2.7V |
| 56 | Power budget table for primary mode | PASS | §6 — 5 consumers with current, duty cycle, average power. Total ~1.3W. |
| 57 | Target battery life stated | PASS | §6 — ~7 hours alkaline, target ≥2 hours met |
| 58 | Back-of-envelope calculation done | PASS | §6 — 9 Wh ÷ 1.3W = ~7 hours |
| 59 | Charging method specified | N/A | User-replaceable AA cells, no charging circuit |
| 60 | [HW↔SW] FW role in power management defined | PASS | §6 — firmware detects low battery via ADC, reduces fan speed, stops cycling on critical |
| 61 | [HW↔SW] Radio duty cycle specified | N/A | No radio |

## Connectivity

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 62–67 | All connectivity items | N/A | §7 — "Not applicable. Bubbler V1 has no wireless or wired data connectivity." Justified. |

## Key Decisions

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 68 | At least 3 non-obvious decisions documented | PASS | §8 — 8 decisions covering sensing, MCU, actuators, fan, battery, position sensing, enclosure, algorithm |
| 69 | Options considered, chosen approach, rationale | PASS | Every decision lists 2–4 options with rationale |
| 70 | Consequences and risks stated | GAP | Decisions list rationale but don't explicitly state downstream consequences/risks of each choice. Minor — the risks are covered in §10. |
| 71 | 3 decisions that would force redesign if reversed | PASS | Implicit: (1) strain gauge sensing — reversing requires different sensor + signal chain + firmware, (2) geared DC motor — reversing to servo changes H-bridge circuit + firmware velocity control, (3) 3-part enclosure — reversing requires new mold tooling |
| 72 | [HW↔SW] HW/SW tradeoff decisions explicit | PASS | Decision 6 (open-loop + end-stops vs. encoder) trades HW cost for FW calibration complexity. Decision 8 (hill-climbing) trades algorithm sophistication for flash/RAM. |

## Constraints

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 73 | Required certifications listed | PASS | §9 — FCC Part 15 Class B, CE (EMC + RoHS), toy safety TBD |
| 74 | Operating environment defined | PASS | §9 — 5–40°C operating, IPX4 electronics, UV-stabilized enclosure |
| 75 | Target BOM cost stated | PASS | §9 — $12.80 estimated, target <$15 for sub-$50 retail |
| 76 | Target production volume stated | PASS | §9 — 10k+ units |
| 77 | Key schedule milestones listed | PASS | §9 Schedule — EVT week 6, DVT week 20, PVT week 24, first lot week 28–30 |
| 77a | Schedule with durations, dependencies and critical path | PASS | §9 Schedule — 14 tasks, Gantt chart; critical path runs through enclosure tooling, 30 weeks |
| 78 | Third-party dependencies identified | PASS | Implicit — COTS motors, fans, HX711, STM32 availability. No single-source risks identified. |
| 78a | Long-lead and high-MOQ parts identified | PASS | §9 BOM — enclosure (10–12 wk, tooling plus first articles) sets the critical path; geared motor is next (6–8 wk, 5k MOQ). MCU has a pin-compatible alternate. |
| 79 | [HW↔SW] App store constraints | N/A | No app |
| 80 | [HW↔SW] Manufacturing test requirements | PASS | §4.1 — SWD programming via Tag-Connect. §4.2 — factory tare calibration of strain gauge at boot. |

## Open Questions and Risks

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 81 | All questions have owner and target date | PASS | §10 — 6 questions with deadlines (relative to milestones) |
| 82 | High-impact risks have mitigation plans | PASS | §10 — 6 risks with likelihood, impact, and mitigation strategies |
| 83 | No question open >2 weeks without progress | PASS | Fresh document — all newly opened |
| 84 | [HW↔SW] Cross-domain risks flagged | PASS | §10 #1 (motor vibration tears film — HW problem mitigated by FW velocity profiles), #2 (noisy force signal — HW noise mitigated by FW filtering) |

## Overall Quality

| # | Item | Status | Notes |
| --- | --- | --- | --- |
| 85 | No section is placeholder-only | PASS | Every section has real content with specific numbers |
| 86 | Consistent terminology | PASS | Consistent use of "blow position," "dip position," "force curve," "cycle" throughout |
| 87 | Diagrams match text | PASS | Block diagram matches §5 interface table; arrangement diagram matches §4.1 component specs |
| 88 | [HW↔SW] Cross-domain consistency check | PASS | Power budget matches component specs (fan 300–500mA, motor 150–250mA); BOM matches component choices; firmware modules match HW interfaces (9 modules map to 8 interfaces) |
| 89 | [HW↔SW] Cross-domain review | N/A | Needs human review |
| 90 | Open questions resolved or carried as TBDs | PASS | §10 — all carried with deadlines |

## Summary

| Category | Pass | N/A | Gap | Total |
| --- | --- | --- | --- | --- |
| Vision and Context | 6 | 0 | 0 | 6 |
| User Scenarios | 5 | 1 | 0 | 6 |
| System Architecture | 9 | 0 | 0 | 9 |
| Hardware | 7 | 0 | 0 | 7 |
| Firmware | 5 | 0 | 1 | 6 |
| Companion App | 0 | 6 | 0 | 6 |
| Cloud / Backend | 0 | 4 | 0 | 4 |
| Interfaces | 6 | 2 | 0 | 8 |
| Power Architecture | 6 | 2 | 0 | 8 |
| Connectivity | 0 | 6 | 0 | 6 |
| Key Decisions | 4 | 0 | 1 | 5 |
| Constraints | 8 | 1 | 0 | 9 |
| Open Questions | 4 | 0 | 0 | 4 |
| Overall Quality | 5 | 1 | 0 | 6 |
| Total | 65 | 23 | 2 | 90 |

### Gaps to Address

FW versioning scheme (#34): Add a firmware version byte or struct in flash for factory traceability (e.g., {major, minor, build} readable via SWD during production test).

Decision consequences (#70): §8 decisions state rationale but not explicit downstream consequences. The risks are captured in §10, so this is a formatting gap rather than a content gap.

### N/A Justification

23 items are N/A — covering companion app (6), cloud/backend (4), connectivity (6), and individual items for offline scenarios (standalone device), security boundaries (no attack surface), charging (user-replaceable cells), radio duty cycle (no radio), FW↔App/App↔Cloud interfaces (no app/cloud), app store constraints (no app), and cross-domain review (needs human). All are justified by V1 being a fully standalone device with no connectivity.

### Gate Decision

PASS — proceed to next phase. The 2 gaps are minor (versioning is a one-line addition, consequences are already covered in §10). None block the product definition from being actionable.